|  | Simulation of Cantilever Bracket 1-2 Date: Sunday, October 22, 2023 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 5 Sensor Details 6 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 10. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Cantilever Bracket 1-2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line3; Solid Body; Mass:0.828668 kg Volume:0.000298082 m^3 Density:2,780 kg/m^3 Weight:8.12095 N; C:\Users\jakob\software\TU\23FL\CAD\HW5.1\Models and Drawings\Cantilever Bracket 1-2.SLDPRT Oct 22 19:27:23 2023.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 77 Fahrenheit. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\jakob\software\TU\23FL\CAD\HW5.1\Models and Drawings).

# Units Unit system:; SI (MKS). Length/Displacement; m. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 2024-T4. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 3.25e+08 N/m^2. Tensile strength:; 4.7e+08 N/m^2. Elastic modulus:; 7.24e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,780 kg/m^3. Shear modulus:; 2.8e+10 N/m^2. Thermal expansion coefficient:; 2.32e-05 /Kelvin.; SolidBody 1(Split Line3)(Cantilever Bracket 1-2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -6,500; 8.39233e-05; 6.10352e-05; 6,500. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 edge(s). Reference:; Edge< 1 >. Type:; Apply force. Values:; ---, ---, -6,500 N..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points for High quality mesh; 16 Points. Element Size; 6.68184 mm. Tolerance; 0.334092 mm. Mesh Quality; High. ## Mesh information - Details Total Nodes; 10720. Total Elements; 6514. Maximum Aspect Ratio; 3.4521. % of elements with Aspect Ratio < 3; 99.9. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:01. Computer name:; JW-MACHINE.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -6,500; 8.39233e-05; 6.10352e-05; 6,500. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.00134284; -1.39495e-05; 0.000122687; 0.00134851. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; SX: X Normal Stress; -57,498,496.000N/m^2 Node: 370; 99,194,136.000N/m^2 Node: 34. Cantilever Bracket 1-2-Static 1-Stress-Stress1. Name; Type; Min; Max. Copy[ 1 ] Stress1; VON: von Mises Stress; 7,766.246N/m^2 Node: 2442; 92,937,448.000N/m^2 Node: 34. Cantilever Bracket 1-2-Static 1-Stress-Copy[ 1 ] Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00m Node: 77; 2.040e-03m Node: 130. Cantilever Bracket 1-2-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 5.016e-07 Element: 1837; 9.861e-04 Element: 4066. Cantilever Bracket 1-2-Static 1-Strain-Strain1.

# Conclusion