|  | Simulation of Quiz 6 Date: Thursday, November 2, 2023 Designer: Jakob Werle Study name: Static 2 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 3 Study Properties 4 Units 4 Material Properties 5 Loads and Fixtures 6 Connector Definitions 7 Interaction Information 8 Mesh information 9 Sensor Details 9 Resultant Forces 9 Beams 10 Study Results 11 Conclusion 14. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Quiz 6 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Bolt Hole x 2; Solid Body; Mass:2.38417 lb Volume:8.57059 in^3 Density:0.27818 lb/in^3 Weight:2.38255 lbf; D:\NOT SOFTWARE\TU23FL-CAD-Q6\Hitch 7-3.SLDPRT Nov 2 14:30:02 2023. Cut-Extrude1; Solid Body; Mass:3.61118 lb Volume:12.7334 in^3 Density:0.283599 lb/in^3 Weight:3.60873 lbf; D:\NOT SOFTWARE\TU23FL-CAD-Q6\Hitch Channel 7-3.SLDPRT Nov 2 14:30:02 2023.

# Study Properties Study name; Static 2. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (D:\NOT SOFTWARE\TU23FL-CAD-Q6).

# Units Unit system:; English (IPS). Length/Displacement; in. Temperature; Fahrenheit. Angular velocity; Hertz. Pressure/Stress; psi.

# Material Properties Model Reference; Properties; Components. Name:; Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 89,984.6 psi. Tensile strength:; 104,982 psi. Elastic modulus:; 3.04579e+07 psi. Poisson's ratio:; 0.28. Mass density:; 0.27818 lb/in^3. Shear modulus:; 1.1458e+07 psi. Thermal expansion coefficient:; 7.22222e-06 /Fahrenheit.; SolidBody 1(Bolt Hole x 2)(Hitch 7-3-1). Curve Data:N/A. Name:; ASTM A36 Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 36,259.4 psi. Tensile strength:; 58,015.1 psi. Elastic modulus:; 2.90075e+07 psi. Poisson's ratio:; 0.26. Mass density:; 0.283599 lb/in^3. Shear modulus:; 1.15015e+07 psi.; SolidBody 1(Cut-Extrude1)(Hitch Channel 7-3-1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(lbf); 440; 220; 5.4885e-05; 491.935. Reaction Moment(lbf.in); 0; 0; 0; 0.. Load name; Load Image; Load Details. Remote Load (Distributed connection)-1; Entities:; 1 face(s). Connection Type:; Distributed. Weighting Factor:; Default (Constant). Coordinate System:; Coordinate System1. Translational Components:; 440 lbf,-220 lbf,---. Rotational Components:; ---,---,---. Reference coordinates:; 0 1.616 0 in..

# Connector Definitions Pin/Bolt/Bearing Connector Model Reference; Connector Details; Strength Details. Counterbore with Nut-1; Entities:; 2 edge(s). Type:; Bolt(Head/Nut diameter)(Counterbore). Connection Type:; Distributed. Head diameter:; 0.75 in. Nut diameter:; 0.75 in. Nominal shank diameter:; 0.5 in. Material name:; Custom. Young's modulus:; 3e+07 psi. Poisson's ratio:; 0.3. Preload (Torque):; 1,340 lbf.in. Friction Factor (K):; 0.2. Tight Fit:; No.; No Data. Connector Forces Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (lbf); -13,434; -4.1562e-12; 0; 13,434. Shear Force (lbf); 0; -33.391; -37.622; 50.302. Bending moment (lbf.in); -1.0492e-14; 33.912; -21.093; 39.937.. Counterbore with Nut-2; Entities:; 2 edge(s). Type:; Bolt(Head/Nut diameter)(Counterbore). Connection Type:; Distributed. Head diameter:; 0.75 in. Nut diameter:; 0.75 in. Nominal shank diameter:; 0.5 in. Material name:; Custom. Young's modulus:; 3e+07 psi. Poisson's ratio:; 0.3. Preload (Torque):; 1,340 lbf.in. Friction Factor (K):; 0.2. Tight Fit:; No.; No Data. Connector Forces Type; X-Component; Y-Component; Z-Component; Resultant. Axial Force (lbf); -13,429; -4.1546e-12; 0; 13,429. Shear Force (lbf); 0; -97.257; 40.412; 105.32. Bending moment (lbf.in); 3.8267e-15; -12.369; -46.708; 48.318..

# Interaction Information Interaction; Interaction Image; Interaction Properties. Local Interaction-1; Type:; Contact interaction pair. Entities:; 2 face(s). Advanced:; Surface to surface.. Contact/Friction force Components; X; Y; Z; Resultant. Contact Force(lbf); -2.2488E-11; 0; 0; 2.2488E-11.. Global Interaction; Type:; Bonded. Components:; 1 component(s). Options:; Independent mesh..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 0.554582 in. Minimum element size; 0.110916 in. Mesh Quality; High.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; 440; 220; 5.4885e-05; 491.935. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf; -0.0101975; -0.000584553; -0.000156307; 0.0102155. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; lbf.in; 0; 0; 0; 8.85075e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 5.750e+01psi Node: 4974; 6.705e+04psi Node: 4370. Quiz 6-Static 2-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00in Node: 3493; 1.805e-02in Node: 41. Quiz 6-Static 2-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.092e-06 Element: 2367; 1.554e-03 Element: 6477. Quiz 6-Static 2-Strain-Strain1. Name; Type; Min; Max. Stress2; CP: Contact Pressure; 0.000psi Node: 1; 44,417.719psi Node: 687. Quiz 6-Static 2-Stress-Stress2.

# Conclusion